DIRECTION OF SOUND DETERMINATION

The direction of sound is determined by measuring the difference

in time that a sound strikes three microphones placed at equal

intervals around the base.  Each microphone is connected to a

voltage comparator.  These are all adjusted to give an output

pulse at exactly the same sound volume.  The comparator outputs

are used to trigger three Z80-CTC channels.  The CTC's operate in

timer mode with an external trigger.  When triggered, the timers

start decrementing from a count of 256 down to zero.  The system

clock is divided by 16 to yield a count interval of 4us.  The

speed of sound in air is 1085 feet per second, which is 0.05208

inches every 4us.

When the sound passes the first microphone, its timer starts

decrementing.  When the timer reaches zero, it interrupts the

CPU.  At this time, the current count values of all three timers

are read and the number of the interrupting timer is noted.  The

second timer to be triggered will be the one with the next lowest

remaining count.

The difference in counts between the first and second timers

provides the length of the B side of a right triangle (fig-1).

The distance between the microphones is a known constant, and is

the length of line C.  Using the Pythagorean theorem, the length

of line A can be calculated.  Next, angle AC is calculated using

the COSINE law.  In figure-1, lines B and Z are parallel, and

lines C and X are parallel.  Therefore angles BC and XZ are

equal.  Angles AB and XY are equal, since they are both right

angles.  Angles AC and YZ must be equal since the sum of the

angles in both triangles must total 180 degrees (Pi radians).

Since angle AC is known, angle YZ is also known.  Angle YZ is the

difference between the direction of sound and a line through the

mid-point between two microphones (line Y).

Rather than go through these lengthy calculations for each

direction-of-sound measurement, a table was generated (by a BASIC

program) which equates each possible value for line X with the

number of motor steps between the first microphone and the

direction of sound.  A simple table look-up converts the timer

counts to stepper counts.  In addition to the CPU time saved,

this approach also saves the effort of coding floating point and

trig function routines.

Since there are three microphones, and the angle between the

microphone and the midpoint is calculated, there are six places -

hextants - (like quadrants excecpt there are six of them) around

the circle where that angle could belong.  After the angle is

calculated (in stepper motor units), it must be placed in the

proper hextant.  If TOTAL is the number of steps required to make

a full revolution, then the number if steps between microphones

is TOTAL/3, and the number of steps in a hextant is TOTAL/6.  In

order to find the proper hextant, the step value for the

calculated angle must be added to or subtracted from the step

value for the nearest microphone.  The following table shows the

absolute position value obtained, depending on which microphone

was first and which microphone was second to trigger.  X is the

value for the calculated angle.

FIRST                             SECOND

MIC-0             MIC-1          MIC-2

MIC-0           -----               X           TOTAL-X

MIC-1         TOTAL/3-X           -----         TOTAL/3+X

MIC-2         TOTAL*2/3+X      TOTAL*2/3-X       -----

############################